- 1,Fail to visit IP camera via IE browser

Possible Reason 1: Network is disconnected.

Solution: Connect your PC to network, checking whether it works properly or not. Check whether there is cable failure,power problem,network failure or PC virus, until PCs can be connected with the command of Ping.

Possible reason 2: IP Address has been occupied by other devices

Solution: Stop the connection between IP camera and Network, and connect the IP camera to PC directly, change the IP address if IP camera,please pay attention to the IP address collision.

Possible reason 3: IP addresses are in different subnets.

Solution: Check IP address, subnet masking address and gateway of the         PC or server,Change the gateway of PC or add a new IP to PC and share the same sub network with IP camera.

Possible reason 4: Physical address of network conflict with IP camera

Solution: modify the physical address of IP camera.

Possible Reason 5: Web port has been modified

Solution: Contact with Network Administrator to obtain related information.

Possible Reason 6: Unknown

Solution: Press RESET to restore default settings then connect it again, the default IP address is 192.168.1.88, subnet mask is 255.255.255.0

- 2,No video image displayed in IE browser

Possible reason: ActiveX not installed

Solution: ActiveX must be installed when visiting IP camera for the first time via Internet Explore.

How to install: Visit IP camera, click [file], and download dialog will pop up, select [Run] or [Save] to download. Please reference the ActiveX install part to install the ActiveX.

3, Can not install ActiveX correctly.

Possible reason: IE with high security level

Solution: